Job Safety Analysis

|  | Page: 1 of 1 | JSA Reference #: |  |  | Date: 11/14/13 |  | New: X Revised: |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Job/Task Description: HIGH-VISIBILITY CLOTHING REQUIREMENTS |  |  |  | Analysis by: CIRSA Loss Control |  |  |
|  | Department(s): All Affected |  |  | Section(s): All Applicable | Revised by: N/A |  |  |
|  | Additional Information: This JSA applies to any person who, as part of their daily job tasks and/or as part of periodic assignments related to their job tasks must be present within a city, county or state right-of-way. The requirements for this clothing are established through the Manual on Uniform Traffic Control Devices (MUTCD). |  |  |  |  | Approved by: CIRSA Loss Control |  |
| ## Sequence of Job/Task | ## Potential Hazards |  | ## Recommended Action(s), Procedure(s) and/or Personal Protective Equipment (PPE) |  |  |  |  |
| Ensure adequate training has been conducted. | Improper training may lead to the failure to wear approved high visibility clothing that may result in injury or death. |  | A thorough review of the MUTCD, Sections 6D.03, Worker Safety Considerations and 6E.02, High-Visibility Safety Apparel may accomplish this task. Note: The requirement to wear high-visibility Clothing also applies to emergency responders, such as law enforcement, fire and other EMS personnel. Exceptions are listed in sections 6D.03 and 6E.02 of the MUTCD. |  |  |  |  |
| Arrive Onsite – Within city, county or state rights-of-way. | Improper high visibility clothing or the failure to wear adequate high visibility clothing may contribute to vehicular traffic not seeing the individual(s) and may result in injury or death. |  | Don an ANSI/ISEA 107-2004 approved (or the most current edition) class 2 or 3 high visibility vest, shirt, jacket, rainwear, coat or other ANSI class 2 or 3 garment prior to exiting the vehicle or immediately upon exiting the vehicle. ANSI Class 2 or Class 3 garments are required for daytime and night-time use. Class 3 garments are recommended This includes personnel that may visit the “work zone” temporarily, such as supervisory and management and/or vendors. Time is not a factor in determining high-visibility clothing usage. Note: High-visibility t-shirts and other bright colored clothing that do not display retro-reflective material on the garment do not meet the ANSI requirements unless falling under one of the exceptions listed below. General Recommendation – Many states use 15 feet as their standard right-of-way and therefore, employees should consider wearing their high-visibility clothing anytime they are within the 15’ distance. However, some city or county rights-of-way may be larger and high-visibility clothing should be worn at all times to ensure compliance. |  |  |  |  |
| Exceptions. | Improper high visibility clothing or the failure to wear adequate high visibility clothing may contribute to vehicular traffic not seeing the individual(s) and may result in injury or death. |  | These exceptions only apply to the specific industries mentioned and should not be transferred or adopted for convenience. a) Workers operating machinery or equipment in which loose clothing could become entangled during operation should wear fitted high-visibility safety apparel, Examples include t-shirts or button up shirts that can be tucked in. b) Workers inside the bucket of a bucket truck, who may be exposed to electrical hazards, are not required to wear high-visibility safety apparel while inside the bucket but must don apparel when performing other traffic control tasks when outside of the bucket. c) When other industry apparel safety standards require workers to wear apparel that is inconsistent with this directive, the prevailing standards shall apply. Examples include utility workers working specifically with electricity and fire/EMS/police personnel performing life saving duties in/near fires, vehicle extractions, etc. The MUTCD via sections 6D.03 and 6E.02 list the specific MUTCD language regarding these exceptions. |  |  |  |  |
| Safety Task Complete. | None. |  | None. |  |  |  |  |